## FPGA-Based Smart Security Locker with Camera Monitoring and Telegram Alerts

# Internship Project Documentation

# Submitted By

# Sinchana
                                                     Pranam U

# Internship Organization

# VIVEQA
Anmaya Technologies

# Table of Contents

- Project Overview

- Project Objectives

- Problem Statement

- Existing System

- Proposed System

- Design and Architecture

- Hardware Components

- Software Requirements

- Working Principle

- System Flow

- Module Description

- Build Instructions

- Run Instructions

- Testing Procedure

- Test Results

- Conclusion

- References

# 1. Project Overview

### This project presents an FPGA-Based Smart Security Locker with Camera Monitoring and Telegram Alerts. The system is designed to provide secure access control using a password-based authentication mechanism implemented on an Artix-7 FPGA. Users enter a password through a 4×4 keypad. If the entered password is correct, the FPGA unlocks the locker using a servo motor, activates a green LED, generates a short buzzer confirmation, and displays the success status on a MAX7219-based seven-segment display.

### If an incorrect password is entered, the FPGA activates a red LED and buzzer, then sends a UART command to the onboard ESP32-C3 module. The ESP32-C3 communicates with the ESP32-CAM over Wi-Fi. The camera captures an image of the unauthorized user and immediately sends it to a Telegram bot, providing real-time intrusion alerts.

### The system combines FPGA-based digital design, embedded systems, wireless communication, and IoT technology to create an intelligent locker suitable for homes, offices, laboratories, and security-sensitive environments.

# 2. Project Objectives

### • Design a secure FPGA-based locker system.

### • Authenticate users using a keypad.

### • Unlock the locker using a servo motor.

### • Generate visual and audio feedback.

### • Capture intruder images during unauthorized access.

### • Send captured images to Telegram.

### • Demonstrate FPGA and IoT integration.

# 3. Problem Statement

### Traditional lockers provide only mechanical security and cannot notify the owner during unauthorized access attempts. This project aims to improve security by combining FPGA-based password authentication with real-time camera monitoring and Telegram notifications.

# 4. Existing System

### Traditional lockers use mechanical keys or simple electronic locks. These systems cannot capture images of intruders, send remote alerts, or provide real-time monitoring. Passwords are usually fixed and no notification mechanism is available.

# 5. Proposed System

### The proposed smart locker uses an FPGA as the primary controller. A keypad is used for password entry. Correct passwords unlock the locker, while incorrect passwords activate an alarm and trigger image capture through the ESP32-CAM. The captured image is sent instantly to Telegram using Wi-Fi.

# 6. Design and Architecture

# 7. Hardware Components

### • Artix-7 FPGA Development Board

### • ESP32-C3 Mini

### • ESP32-CAM

### • 4×4 Keypad

### • Servo Motor

### • Buzzer

### • Green LED

### • Red LED

### • MAX7219 Seven Segment Display

### • Wi-Fi Hotspot

# 8. Software Requirements

### • Vivado Design Suite

### • Arduino IDE

### • Git

### • Telegram Bot API

# 9. Working Principle

### User enters the password through the keypad.

### FPGA verifies the password.

### Correct password:

### • Servo unlocks.

### • Green LED turns ON.

### • Short buzzer confirmation.

### • Seven-segment display updates.

### Incorrect password:

### • Red LED turns ON.

### • Alarm buzzer activates.

### • FPGA sends UART command.

### • ESP32-C3 receives command.

### • ESP32-CAM captures image.

### • Image is sent to Telegram.

# 10. System Flow

# 11. Module Description

### keypad_decoder.v

### Reads keypad inputs and generates the corresponding key code.

### password_fsm.v

### Stores password digits, verifies authentication, and generates unlock or alarm signals.

### uart_message.v

### Generates UART messages ("G" and "D") for communication with the ESP32-C3.

### uart_tx.v

### Serially transmits UART data to the ESP32-C3.

### servo_pwm.v

### Generates PWM signals for locking and unlocking the servo motor.

### buzzer_controller.v

### Produces confirmation beeps and alarm patterns.

### password_display.v

### Displays password entry and authentication status.

### seg_display.v

### Drives the MAX7219 seven-segment display.

### smart_locker_top.v

### Integrates all hardware modules into the complete system.

# 12. Build Instructions

### Open Vivado.

### Create a new RTL project.

### Add all Verilog source files.

### Add the XDC constraints file.

### Run Synthesis.

### Run Implementation.

### Generate Bitstream.

### Program the FPGA.

### Upload ESP32-C3 firmware.

### Upload ESP32-CAM firmware.

# 13. Run Instructions

### Power the FPGA board.

### Power the servo motor.

### Connect ESP32-C3 and ESP32-CAM to the same Wi-Fi network.

### Verify Telegram Bot configuration.

### Enter a password using the keypad.

### Observe the system response.

# 14. Testing Procedure

| Test Case | Description | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- |
| TC-01 | Correct Password Authentication | Servo motor unlocks, Green LED turns ON, seven-segment display shows 1111, UART transmits 'G', no alarm is generated, and no image is captured. | System operated as expected. | Pass |
| TC-02 | Incorrect Password Authentication | Red LED turns ON, alarm buzzer is activated, seven-segment display shows 9999, UART transmits 'D', ESP32-C3 triggers the ESP32-CAM, captured image is sent to Telegram. | System operated as expected. | Pass |
| TC-03 | Password Clear Function | Pressing the F key clears the previously entered password, resets the display, and prepares the system for a new password entry. | System operated as expected. | Pass |
| TC-04 | UART Communication | FPGA successfully transmits 'G' for correct authentication and 'D' for incorrect authentication. ESP32-C3 correctly receives both commands. | UART communication verified successfully. | Pass |
| TC-05 | Telegram Notification | ESP32-CAM captures the intruder's image and successfully sends it to the configured Telegram bot after an incorrect password attempt. | Image successfully received on Telegram. | Pass |

# 15. Test Results

### The FPGA successfully authenticated users using keypad input. Correct passwords unlocked the servo motor, while incorrect passwords triggered the ESP32-CAM to capture images and send them to Telegram. The system successfully demonstrated FPGA, UART, Wi-Fi, and IoT integration.

Figure 15.1
Hardware Setup:

Figure 15.2
Correct Password:

Figure 15.3
Wrong Password:

Figure 15.4
Telegram Alert:

Figure 15.5
Simulation:

# 16. Conclusion

The FPGA-Based Smart Security Locker successfully combines password authentication, servo-based access control, camera monitoring, and Telegram notifications into a reliable security solution. The project demonstrates effective integration of FPGA hardware design with embedded systems and IoT technologies to provide enhanced security and real-time intrusion monitoring.

# 17. References

### • Xilinx Vivado Documentation

### • Artix-7 FPGA User Manual

### • ESP32-C3 Technical Reference

### • ESP32-CAM Documentation

### • Telegram Bot API Documentation